# PDPNRMCV2 PaDelhi™ Panoramic Padel Court System and Accessories

# SECTION 11 68 23.33 	Athletic Field Equipment – Tennis Equipment

# (Formally Section 11480)

PART 1	GENERAL

1.01		WORK INCLUDED

A.		Provide all equipment and materials, and do all work necessary to furnish

and install the athletic equipment, as indicated on the drawings and as

specified herein.  Athletic equipment shall include, but not be limited to:

- PDPNRMCV2 PaDelhi™ Panoramic Padel Court System and Accessories

1.02		RELATED WORK

A.		Examine contract documents for requirements that affect work of this

section.  Other specification divisions and sections that directly relate to
                       	the work of this section include, but are not limited to:

1.  Division 03 – Concrete; Sections: Cast-in-Place Concrete

2.  Division 31 – Earthwork; Sections: Excavation and Backfill and
                                        Establishment of Sub-Grade Elevations

3.  Division 32 – Exterior Improvements; Sections: Athletic and
                                                                Recreational Surfacing, Concrete and Asphalt

1.03		REFERENCES

A.		Comply with applicable requirements of the following standards. Where

these standards conflict with other specified requirements, the most

restrictive requirements shall govern.

1.  International Padel Federation (FIP)

2.  United States Padel Association (USPA)

3.  National Federation of State High School Associations (NFHS)

4.  World Athletics (WA), Formerly International Association of
     Athletics Federations (IAAF)

5.  National Collegiate Athletic Association (NCAA)

6.  American Sports Builders Association (ASBA)

7.  Manufacturer’s Data and Recommended Installation

Requirements

1.04		SUBMITTALS

A.		Manufacturers Product Data

1.  Provide manufacturers product data prior to actual field

installation work, for Architects or Owners representatives

review.

B.		Shop Drawings

1.  Provide drawings of the manufacturers recommended

installation and foundation requirements prior to actual field

installation work, for Architects or Owners representatives

review.

1.05		QUALITY ASSURANCE

A.		Manufacturers warranties shall pass to the Owner and certification made

that the product materials meet all applicable grade trademarks or conform 		to industry standards and inspection requirements. The Manufacturer
                        shall have a current American Sports Builders Association (ASBA)
                        Supplier Certificate of Distinction designation. The Manufacturer shall be

a current United States Padel Association (USPA) Approved and Endorsed Padel Court Manufacturer/Builder.

1.06		PRODUCT DELIVERY AND STORAGE

A.		Materials delivered to the site shall be examined for damage or defects in 			shipping.  Any defects shall be noted and reported to the Owners 				representative.  Replacements, if necessary, shall be immediately re-			ordered, so as to minimize any conflict with the construction schedule.  			Sound materials shall be stored above ground under protective cover or 			indoors so as to provide proper protection.

PART 2	PRODUCTS

2.01		PDPNRMCV2 PaDelhi™ Panoramic Padel Court System and
            Accessories

A. 		BASE:  PDPNRMCV2 PaDelhi™ Panoramic Padel Court System and
                                     Accessories as Manufactured and/or Supplied by:

Sportsfield Specialties, Inc.

P.O. Box 231

41155 State Highway 10

Delhi, NY 13753

p. 888-975-3343

www.sportsfield.com

B.		COMPONENTS:

- Base Plates and Reinforcement Gussets Fabricated of 1/2” Thick Welded Steel and Anchored with 1/2" x 6"L, KH-EZ CRC Hilti Screw Anchor, Mechanically Galvanized

- Eight (8) 3 Meter Posts Fabricated of 2” x 4” 11 Gauge Steel Rectangular Tube with Welded Reinforcement Gusset

- Four (4) 3 Meter Light Posts Fabricated of 4” x 4” x 1/4” Wall Steel Square Tube with Welded Reinforcement Gusset

- Four (4) 4 Meter Corner Posts Fabricated of 5” x 5” x 1/4” Wall Steel Square Tube with Welded Reinforcement Gussets

- Two (2) Top Cross Members Fabricated of 2” x 6” x 3/8” Wall Steel Rectangular Tube with Bolted Inner Splice Connection and Welded End Caps

- Four (4) Top Side Members Fabricated of 2” x 6” x 3/8” Wall Steel Rectangular Tube and Welded End Caps

- Eight (8) Bottom End and Side Cross Members, 2” x 4” x 1/8” Wall Aluminum Rectangular Tube with Integrated Drainage Channels and Anchored with 1/2" x 3"L, KH-EZ CRC Hilti Screw Anchor, Mechanically Galvanized

- 2” x 2” Square Mesh, 1.844” x 1.844” Opening, 0.156” Thick (8-1/4 Gauge) Wire Diameter, Carbon Steel with Welded 1” x 2” 11 Gauge Steel Rectangular Tube Frame and Cross Bracing

- Mesh Panel Support Brackets Fabricated of 11 Gauge Steel

- Support Structure and Mesh Panels Have a Two (2) Coat Powder Coated Process Consisting of Zinc Gray Epoxy Primer and Super Durable Black Finish with Enhanced Resistance to UV Fade and Corrosion

- Nineteen (19) 1/2” (12mm) Clear Tempered Glass Panels, 2 Meters Wide x 3 Meters High, Polished Edge, Includes Ten (10) Factory Pre-Installed Holes and Attachment Hardware

- Top Glass Retainer Brackets Fabricated of Bent 3/16” Steel

- Bottom Glass Retainer Brackets Fabricated of Bent 11 Gauge Steel

- Middle and Corner Glass Support Brackets Fabricated of 3/8” (10mm) Clear Polycarbonate

- Four (4) Light Extension Arms Fabricated of 2” x 4” 11 Gauge Steel Rectangular Tube with Welded Support Gusset

- Four (4) 400-Watt LED Light Fixtures with Attachment Slip Fitter Coupler (Meets Both Outdoor and Indoor Padel Court Lighting Specifications and Requirements) and 30’L 14 Gauge, Three (3) Conductor (White, Black, and Green) Wire Lead Connectors, Main Electrical Connections by Others, Optional Dimming Feature Available with Different Wiring Configuration

- Surface Mount Padel Court Posts – Set of Two (2)

  - 250mm x 250mm x 10mm Welded Steel Base Plate with Four (4) 12mm Diameter Mounting Holes, Installed with Provided Anchoring Hardware

  - 36”H, 80mm x 80mm Square Steel Reinforced Welded Uprights

  - Powder Coated Black Finish

  - Top Access Internal Wind Winch System

  - Includes Crank Handle and Aluminum Lacing Bars

- Padel Net

  - 5mm High Tenacity, UV Stabilized, Rot Proof Polypropylene Black Fiber

  - 0.92m High x 10m Long

  - White PVC Top Net Tape with Four (4) Rows of Reinforced Stitching

  - Black PVC Sides and Bottom Net Tape

  - Includes 5mm Diameter Galvanized Steel Top Net Cable with Net Post Attachment Loops on Each End and Two (2) Fiberglass Net Lacing Rods

- Synthetic Turf:

a.  Texturized Monofilament Polyethylene Fibers

b.   Pile Height: 12mm

c.   Backing: Latex

d.   Color: Standard is Blue, Green, or Terracotta with
                  Inlaid White Lines, Various Additional Colors
                  Available Upon Request

e.   Includes Seam Tape and Adhesive

f.   Infill with Provided 30-50 Round Sand

- Included Accessories:

  - Model Specific Hardware Kit and Installation Instructions

- Professionally Pre-Engineered Structural Design to Meet ASCE 7-16 (IBC 2018), Risk Category I, Wind Velocity up to 130 mph Exposure B and 106 mph Exposure C (202 mph Exposure B and 165 mph Exposure C if Glass Panels are Removed)

- Professionally Pre-Engineered Stamped Anchoring Foundation Designs to Meet ASCE 7-16 Wind Load  up to 130 mph Exposure B and 106 mph Exposure C (202 mph Exposure B and 165 mph Exposure C if the Glass Panels are Removed), Available for Post Tension Concrete Slab, Reinforced Concrete Grade Beam, and Reinforced Concrete Slab-On-Grade with Thickened Edge, Individual Project Design Stamping is Available Upon Request

- Optional Accessories:

a.   PDLGHTV2 PaDelhi™ Padel Court Lighting System

b.   PDLGV2 PaDelhi™ Padel Court Entrance/Exit Hinged

Locking Gate System, Set of Two (2)

c.   PDPDV2 PaDelhi™ Padel Court Entrance/Exit and Net
                                          Post Padding, Set of Two (2), Various Standard Vinyl
                                          Colors Available, Custom Digitally Printed Graphics
                                          Available

# PART 3 	EXECUTION

3.01		INSTALLATION OF EQUIPMENT

A.  	All PDPNRMCV2 PaDelhi™ Panoramic Padel Court Systems and Accessories shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings.  Concrete anchoring foundations to be determined by others based on local soil conditions and building codes.  Installer should have a minimum of five (5) Padel Court System installations or similar experience in the previous three (3) years.

END OF SECTION