CBBM1830 - ComBox® with Black Mat
SECTION 11 68 33
Athletic Field Equipment
PART 1
GENERAL 

1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein. Athletic equipment shall include, but not be limited to:

1. CBBM1830 - ComBox® with Black Mat
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section. Other specification divisions and sections that directly relate to
                       
the work of this section include, but are not limited to:

1. Division 03 – Concrete; Sections: Cast-in-Place Concrete 
2. Division 26 – Electrical

3. Division 27 – Communications

4. Division 31 – Earthwork; Sections: Excavation and Backfill and Establishment of Sub-Grade Elevations
5. Division 32 – Exterior Improvements; Sections: Athletic and Recreational Surfacing, Concrete and Asphalt
6. Division 33 – Utilities
1.03

REFERENCES

A.

Comply with applicable requirements of the following standards.  Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.

1. National Federation of State High School Associations (NFHS)
2. National Collegiate Athletic Association (NCAA)


3. International Association of Athletics Federations (IAAF)

4. American Sports Builders Association (ASBA)
5. Manufacturers Data and Recommended Installation Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data

1. Provide manufacturers product data prior to actual field installation work, for Architects or Owners representatives review.

B.

Shop Drawings

1. Provide drawings of the manufacturers recommended installation and foundation requirements prior to actual field installation work, for Architects or Owners representatives review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements.

1.06

PRODUCT DELIVERY AND STORAGE

A.
Materials delivered to the site shall be examined for damage or defects in shipping. Any defects shall be noted and reported to the Owners representative. Replacements, if necessary, shall be immediately re-ordered, so as to minimize any conflict with the construction schedule. Sound materials shall be stored above ground under protective cover or indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01
CBBM1830 - ComBox® with Black Mat
A. 
BASE:  CBBM1830 - ComBox® with Black Mat as Manufactured and Supplied by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343
www.sportsfieldspecialties.com
B.
COMPONENTS:
1. CBBM1830 - ComBox® with Black Mat
a. Dimensions: 18”W x 30”L x 14”H
b. Box: 3/16" (0.1875”) Aluminum Construction, Welded Frame with Open Bottom Having the Following Attributes:
i. 1/8" (0.125”) Aluminum Adjustable Main Lid Support Ledge
ii. 3/16" (0.1875”) Aluminum Removable Divider Panel

iii. 1” PVC Drain Stub for Positive Drainage Connection
iv. Leveling Bolts

c. Main Lid: 1/8" (0.125”) Aluminum Construction with the Following Attributes:
i. ½” (0.50”) Recess with Factory Installed Recycled Rubber Black Mat Filler 
ii. Secured with Cam Locks and is Pad Lockable

iii. Includes Wire Feed Cutouts

d. Hand Hole Covers: 1/8" (0.125”) Aluminum Construction with the Following Attributes:
i. Secured with Hex Key 
ii. Includes Wire Feed Cutouts

e. Assembly Hardware
PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  
All Utility Boxes shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings.
    END OF SECTION
Athletic Field Equipment
          11 68 33 
6.21.2019

