SCWJFS50 - Steeplechase Water Jump Pit Forming System, Covers, and Hurdle
SECTION 11 68 33.43   Athletic Field Equipment - Track and Field Equipment
PART 1
GENERAL 
1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein.  Athletic equipment shall include, but not be limited to:

1. SCWJFS50 - Steeplechase Water Jump Pit Forming System, Covers, and Hurdle
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section.  Other specification divisions and sections that directly relate to
                       
the work of this section include, but are not limited to:




1.  Division 03 – Concrete; Sections: Cast-in-Place Concrete 

2.  Division 31 – Earthwork; Sections: Excavation and Backfill and
                                        Establishment of Sub-Grade Elevations



3.  Division 32 – Exterior Improvements; Sections: Athletic and
                                                                Recreational Surfacing, Concrete and Asphalt
1.03

REFERENCES

A.

Comply with applicable requirements of the following standards.  Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.




1.  National Federation of State High School Associations (NFHS)




2.  National Collegiate Athletic Association (NCAA)






3.  World Athletics (Formerly IAAF)

4.  American Sports Builders Association (ASBA)



5.  Manufacturers Data and Recommended Installation 




     Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data




1.  Provide manufacturers product data prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

B.

Shop Drawings




1.  Provide drawings of the manufacturers recommended 




     installation and foundation requirements prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements. The Manufacturer
                        shall have a current American Sports Builders Association (ASBA)
                        Supplier Certificate of Distinction designation.
1.06

PRODUCT DELIVERY AND STORAGE

A.

Materials delivered to the site shall be examined for damage or defects in 


shipping. Any defects shall be noted and reported to the Owners 



representative. Replacements, if necessary, shall be immediately re-


ordered, so as to minimize any conflict with the construction schedule.  


Sound materials shall be stored above ground under protective cover or 


indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01
SCWJFS50 - Steeplechase Water Jump Pit Forming System, Covers, and Hurdle
A. 
BASE:  SCWJFS50 - Steeplechase Water Jump Pit Forming System, Covers, and Hurdle as Manufactured and Supplied by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343
www.sportsfieldspecialties.com
B.

COMPONENTS:  
1. SCWJFS50 - Steeplechase Water Jump Pit Forming System, Covers, and Hurdle:

a. Modular Steeple Chase Water Jump Pit Forming Assembly Fabricated of 1/8” (0.125”) Aluminum with 12’ x 12’ Inside Dimensions
b. Comprised of Left and Right Side and Rear Forming Panels with Double Wall, Aluminum Gusset Reinforced Welded Construction
c. Left and Right Side Forming Panels are Tapered at a 22.4% Slope
d. ½” (0.5”) Exterior Perimeter Bent Aluminum Screed Edge for Terminating Asphalt and Synthetic Track Surface by Others
e. Front Side Spreader Bar to Maintain Squareness of Side Forming Panels, Fabricated of 1/8” (0.125”) Steel Formed Channel
f. Rear Forming Panel Includes Drain Valve Located at the Bottom Center of the Inside Wall with a Removable Access Cover for Maintenance, 2” PVC Outlet is Located on the Exterior Wall of Rear Forming Panel
g. Rear Forming Panel has Pre-Fabricated Ninety Degree (90°) Corner Sections and Includes Two (2) Factory Installed Ground Sleeve Locations for Steeple Chase Water Jump Pit Hurdle Installation and One (1) Drain Valve Extension, Cap and Key Handle Used to Open and Close Drain Valve
h. Left Side and Right Side Cover Ledge Filler Panels are Installed when Covers are Not in Place
i. ½” (0.5”) Recess to Accept Synthetic Track Surface by Others
2. SCWJCVR - Steeplechase Water Jump Pit Cover Set
a. Welded 1/8” (0.125”) Aluminum Panel Cover Set with Recessed Stainless Steel Lift Handles and ½” (0.5”) Recess to Accept Synthetic Track Surfrace by Others
b. Overall Cover Depth: 6”
c. Includes (1) Tapered Wedge Cover for Transitioning to Finish Grade

3. SCWJH - Steeplechase Water Jump Pit Hurdle and Barrier Seal

a. 5” x 5” x 12’L Painted Synthetic Track Surface Covered Steeplechase Water Jump Pit Hurdle that is Height Adjustable for Both Men's (36”H) and Women's (30”H) Steeplechase Events and Includes a Powder Coated White Aluminum Barrier Seal and Aluminum Support Structure
4. Optional:

a. SCHS - Steeplechase Portable Hurdle Set

i. One (1) 5” x 5” x 16’-5”L and Three (3) 5” x 5” x 13’-5”L Painted Synthetic Track Surface Covered Steeple Chase Event Stationary Hurdles that are Height Adjustable for Both Men's (36”H) and Women's (30”H) Steeplechase Events, Set of Four (4) with Removable and Retractable Wheel Kit for Portability and Transport Purposes, 4” x 4” Welded Construction Powder Coated White Aluminum Support Structure
PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  
All Steeplechase Systems shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings. Installer should have a minimum of five (5) Steeplechase System installations or similar experience in the previous three (3) years.
    END OF SECTION
Athletic Field Equipment
        11 68 33.43 – Track and Field Equipment          4.22.2021

