LTJTOB8BL - 8’’ Long and Triple Jump Take-Off Board System with Blanking Lid
SECTION 11 68 33.43   Athletic Field Equipment - Track and Field Equipment
PART 1
GENERAL 
1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein.  Athletic equipment shall include, but not be limited to:

1. LTJTOB8BL - 8’’ Long and Triple Jump Take-Off Board System with Blanking Lid
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section.  Other specification divisions and sections that directly relate to
                       
the work of this section include, but are not limited to:

1. Division 03 – Concrete; Sections: Cast-in-Place Concrete 
2. Division 31 – Earthwork; Sections: Excavation and Backfill and Establishment of Sub-Grade Elevations
3. Division 32 – Exterior Improvements; Sections: Athletic and
Recreational Surfacing, Concrete and Asphalt
1.03

REFERENCES

A.

Comply with applicable requirements of the following standards.  Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.




1.  National Federation of State High School Associations (NFHS)




2.  National Collegiate Athletic Association (NCAA)






3.  International Association of Athletics Federations (IAAF)

4.  American Sports Builders Association (ASBA)



5.  Manufacturers Data and Recommended Installation 




     Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data




1.  Provide manufacturers product data prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

B.

Shop Drawings




1.  Provide drawings of the manufacturers recommended 




     installation and foundation requirements prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements.

1.06

PRODUCT DELIVERY AND STORAGE

A.

Materials delivered to the site shall be examined for damage or defects in 


shipping. Any defects shall be noted and reported to the Owners 



representative. Replacements, if necessary, shall be immediately re-


ordered, so as to minimize any conflict with the construction schedule.  


Sound materials shall be stored above ground under protective cover or 


indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01
LTJTOB8BL - 8’’ Long and Triple Jump Take-Off Board System with Blanking Lid
A. 
BASE:   LTJTOB8BL - 8’’ Long and Triple Jump Take-Off Board System with Blanking Lid as Manufactured and Supplied by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343
www.sportsfieldspecialties.com
B.

COMPONENTS:  
1. LTJTOB8BL - 8’’ Long and Triple Jump Take-Off Board System with Blanking Lid:

a. White Synthetic Polyboard with Textured Surface

i. Dimensions: 8” x 48” x ¾” (0.75”)
ii. Grooved Edges for Slide-on Assembly, No Exposed Fastening Hardware

b. Extruded Aluminum Insert/Spacer
i. 0.120” Wall Thickness

ii. Designed to Accommodate Slide-on Board Assembly
c. Formed Aluminum Blanking Lid

i. 1/8” (0.125”) Aluminum Sheet

ii. ½” (0.5”) Recess to Accept Synthetic Track Surfacing by Others

iii. Includes Leveling Bolts
d. 16 Gauge (0.60”) 304 Stainless Steel Tray
i. Includes 1” PVC Drain Stub (Oriented at Either End) to Ensure Positive Connection to Subsurface Drainage System by Others 
PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  
All Take-Off Board Systems shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings. Installer should have a minimum of five (5) Take-Off Board System installations or similar experience in the previous three (3) years.
    END OF SECTION
Athletic Field Equipment
      11 68 33.43 – Track and Field Equipment              6.14.2019

