# PMFSB – Baseball Pitching Mound Forming System

# SECTION 11 68 33		Athletic Field Equipment

# SECTION 11 68 33.33 	Baseball Field Equipment

# (Formally Section 11480)

PART 1	GENERAL

1.01		WORK INCLUDED

A.		Provide all equipment and materials, and do all work necessary to furnish

and install the athletic equipment, as indicated on the drawings and as

specified herein.  Athletic equipment shall include, but not be limited to:

# PMFSB – Baseball Pitching Mound Forming System

1.02		RELATED WORK

A.		Examine contract documents for requirements that affect work of this

section.  Other specification divisions and sections that directly relate to
                       	the work of this section include, but are not limited to:

1.  Division 03 – Concrete; Sections: Cast-in-Place Concrete

2.  Division 31 – Earthwork; Sections: Excavation and Backfill and
                                        Establishment of Sub-Grade Elevations

3.  Division 32 – Exterior Improvements; Sections: Athletic and
                                                                Recreational Surfacing, Concrete and Asphalt

1.03		REFERENCES

A.		Comply with applicable requirements of the following standards.  Where

these standards conflict with other specified requirements, the most

restrictive requirements shall govern.

1.  National Federation of State High School Associations (NFHS)

2.  National Collegiate Athletic Association (NCAA)

3.  World Athletics (Formerly IAAF)

4.  American Sports Builders Association (ASBA)				5.  Manufacturers Data and Recommended Installation

Requirements

1.04		SUBMITTALS

A.		Manufacturers Product Data

1.  Provide manufacturers product data prior to actual field

installation work, for Architects or Owners representatives

review.

B.		Shop Drawings

1.  Provide drawings of the manufacturers recommended

installation and foundation requirements prior to actual field

installation work, for Architects or Owners representatives

review.

1.05		QUALITY ASSURANCE

A.		Manufacturers warranties shall pass to the Owner and certification made

that the product materials meet all applicable grade trademarks or conform 		to industry standards and inspection requirements.  The Manufacturer shall
                        have a current American Sports Builders Association (ASBA) Supplier
                        Certificate of Distinction designation.

1.06		PRODUCT DELIVERY AND STORAGE

A.		Materials delivered to the site shall be examined for damage or defects in 			shipping.  Any defects shall be noted and reported to the Owners 				representative.  Replacements, if necessary, shall be immediately re-			ordered, so as to minimize any conflict with the construction schedule.  			Sound materials shall be stored above ground under protective cover or 			indoors so as to provide proper protection.

PART 2	PRODUCTS

# 2.01	PMFSB – Baseball Pitching Mound Forming System

# A. 	BASE:  PMFSB – Baseball Pitching Mound Forming System as Manufactured and Supplied by:

Sportsfield Specialties, Inc.

P.O. Box 231

41155 State Highway 10

Delhi, NY 13753

p. 888-975-3343

www.sportsfield.com

# B.		COMPONENTS:

- Baseball Pitching Mound Forming System:

  - Welded Construction, Fabricated of 1/8” Aluminum Sheet Structure and ¼” x 4” Aluminum Bar Outer Ring Rolled to an 18’ Outside Diameter

  - Official Size 18’ Outside Diameter with a Center Section Height of 10” Above Finish Grade and Sloped 1” Vertical Per 1’ Horizontal Per Specifications, Rules and Requirements

  - Five (5) Welded Form Sections; Four (4) Corner Sections and One (1) Center Section with Bolted Connections

  - Notched Cutouts for Rebar Bracing and Stake Holes for Rebar Anchoring (Rebar by Others)

  - Center Section Includes Two (2) ½” Sch. 40 Aluminum Drainage Pipes and Removable Pitching Rubber Tray with Four (4) ¾” Aluminum Round Stock Threaded Studs with 3/8” Stainless Steel Bolts for Pitching Rubber Height Adjustment

  - Two (2) Rear Mound Slope Profile Guides Fabricated of 1/8” Aluminum Sheet with Bolted Connections

  - Includes Replaceable SHBBPB Schutt® Four-Sided Professional Pitching Rubber

  - Includes Synthetic Infill Turf Attachment Ledges

  - Includes Factory Pre-Installed 2” x 4” Synthetic Wood Nailer Boards Around the Outside Perimeter of the Four (4) Corner Form Sections for Synthetic Turf Attachment Purposes

  - Includes ¼” Roll Out Rubber Underlayment for High Traffic Areas

  - Synthetic Turf Replacement Center Section Size is 60”W x 135”L

# PART 3 	EXECUTION

3.01		INSTALLATION OF EQUIPMENT

# A.  	All PMFSB – Baseball Pitching Mound Forming Systems shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings. Installer should have a minimum of five (5) Pitching Mound Forming System installations or similar experience in the previous three (3) years.

END OF SECTION