CD10X16O8 - Cantilever Dugout 10’W x 16’L
SECTION 11 68 33

Athletic Field Equipment
SECTION 11 68 33.33 
Baseball Field Equipment

PART 1
GENERAL 
1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein.  Athletic equipment shall include, but not be limited to:

1. CD10X16O8 - Cantilever Dugout 10’W x 16’L
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section.  Other specification divisions and sections that directly relate to
                       
the work of this section include, but are not limited to:




1.  Division 03 – Concrete; Sections: Cast-in-Place Concrete 

2.  Division 31 – Earthwork; Sections: Excavation and Backfill and
                                        Establishment of Sub-Grade Elevations




3.  Division 32 – Exterior Improvements; Sections: Athletic and
                                                                Recreational Surfacing, Concrete and Asphalt

1.03

REFERENCES

A.

Comply with applicable requirements of the following standards.  Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.




1.  National Federation of State High School Associations (NFHS)




2.  National Collegiate Athletic Association (NCAA)






3.  International Association of Athletics Federations (IAAF)

4.  American Sports Builders Association (ASBA)



5.  Manufacturers Data and Recommended Installation 




     Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data




1.  Provide manufacturers product data prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

B.

Shop Drawings




1.  Provide drawings of the manufacturers recommended 




     installation and foundation requirements prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements.

1.06

PRODUCT DELIVERY AND STORAGE

A.

Materials delivered to the site shall be examined for damage or defects in 


shipping. Any defects shall be noted and reported to the Owners 



representative. Replacements, if necessary, shall be immediately re-


ordered, so as to minimize any conflict with the construction schedule.  


Sound materials shall be stored above ground under protective cover or 


indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01
CD10X16O8 - Cantilever Dugout 10’W x 16’L
A. 
BASE:  CD10X16O8 - Cantilever Dugout 10’W x 16’L as Manufactured and Supplied by:






Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 607-746-1460

www.sportsfieldspecialties.com
B.  

DESIGN CRITERIA:

1. Building Code: ASCE 7-10
2. Maximum Wind Speed Rating: 140mph, Exposure Category C
3. Maximum Ground Snow Load: 60psf
4. Seismic Design: Category E, Ss=1.5g, S1=0.75g
5. Roof Pitch: 3:12
6. Rear Column Offset Overhang: 8”
C.  

COMPONENTS:

1. Cantilever Dugout:

a. Overall Dimensions: 9’ 9-1/8”W x 16’L
b. Structural Columns Fabricated of:
i. 12” x 3” x ¼” (0.25”) Structural Steel Tube with Factory Pre-Drilled 12” x 17” x 1” A36 Steel Base Mounting Plates and 10” x 14” x 3/4” (0.750”) Steel Roof and Column Cap Plates
ii. Fully Welded Construction 
iii. Maximum Allowable Spacing Between Structural Steel Columns is Eight Feet (8’) On-Center 
c. Roof Frame Fabricated of:
i. 8” x 2” x 3/16” (0.1875”) Structural Steel Rectangular Perimeter Tube, 8” x 4” x 1/4” (0.25”) Structural Steel Transverse Roof Tubes and 3” x 2” x 11 Gauge Steel Longitudinal Tubes
ii. Fully Welded Construction
d. Structural Steel Columns and Roof Frame Receive a Powder Coated Primer and Coated Finish, Various Standard and Custom Colors Available
e. Roofing Material is 29 Gauge, Classic Rib® Style Corrugated Metal with J-Channel Drip Cap Installed on Front and Sides, Various Standard Paint Finish Colors Available
f. Structural Columns Attached to Roof Structure with Galvanized Hardware
g. Includes Carbon Steel Anchoring Hardware, Epoxy and Lifting Eye Bolts
h. Model Specific Hardware Kit and Installation Instructions
i. Optional Stamped and Sealed Drawings and Calculations by a Licensed Professional Engineer of Record in the State of Project Location

PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  
All Dugouts shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings. Concrete anchoring foundations to be determined by others based on local soil conditions and building codes. Installer should have a minimum of five (5) Dugout installations or similar experience in the previous three (3) years.

    END OF SECTION
Athletic Field Equipment
11 68 33
7.4.2019

