SHAFIT – Schutt/Hollywood Anchor Access Frame for Infill Turf 
SECTION 11 68 33

Athletic Field Equipment
SECTION 11 68 33.33 
Baseball Field Equipment

PART 1
GENERAL 

1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein. Athletic equipment shall include, but not be limited to:

1. SHAFIT – Schutt/Hollywood Anchor Access Frame for Infill Turf 

1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section. Other specification divisions and sections that directly relate to
                       
the work of this section include, but are not limited to:

1. Division 03 – Concrete; Sections: Cast-in-Place Concrete 
2. Division 31 – Earthwork; Sections: Excavation and Backfill and Establishment of Sub-Grade Elevations
3. Division 32 – Exterior Improvements; Sections: Athletic and Recreational Surfacing, Concrete and Asphalt
1.03

REFERENCES

A.

Comply with applicable requirements of the following standards.  Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.

1. National Federation of State High School Associations (NFHS)
2. National Collegiate Athletic Association (NCAA)


3. International Association of Athletics Federations (IAAF)

4. American Sports Builders Association (ASBA)
5. Manufacturers Data and Recommended Installation Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data

1. Provide manufacturers product data prior to actual field installation work, for Architects or Owners representatives review.

B.

Shop Drawings

1. Provide drawings of the manufacturers recommended installation and foundation requirements prior to actual field installation work, for Architects or Owners representatives review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements.  The Manufacturer
                        shall have a current American Sports Builders Association (ASBA)
                        Supplier Certificate of Distinction designation.
1.06

PRODUCT DELIVERY AND STORAGE

A.
Materials delivered to the site shall be examined for damage or defects in shipping. Any defects shall be noted and reported to the Owners representative. Replacements, if necessary, shall be immediately re-ordered, so as to minimize any conflict with the construction schedule. Sound materials shall be stored above ground under protective cover or indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01

SHAFIT – Schutt/Hollywood Anchor Access Frame for Infill Turf 
A. 

BASE:  SHAFIT – Schutt/Hollywood Anchor Access Frame for Infill Turf 

as Manufactured and Supplied by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343
www.sportsfield.com
B.
COMPONENTS:
1. SHAFIT – Schutt/Hollywood Anchor Access Frame for Infill Turf: 

a. Dimensions: 8-1/16”W x 8-1/16”L x 10”H
b. Box: 1/8" (0.125”) Aluminum Construction, Welded Frame with Open Bottom Having the Following Attributes:
i. 1/8" (0.125”) Aluminum Construction
ii. Integrated Synthetic Infill Turf Attachment Ledge
iii. Infill Retainer System with 1-1/4” (1.25”) Flexible Gasket Seals Specifically Designed for Synthetic Infill Turf Applications

iv. Anchor Base, Anchor Bolts and Anchor Spacer
v. 1” PVC Drain Stub for Positive Drainage Connection
c. Solid Cover: 1/8" (0.125”) Aluminum Construction with the Following Attributes:
i. Infill Retainer System with 1-1/4” (1.25”) Flexible Gasket Seals Specifically Designed for Synthetic Infill Turf Applications

ii. Designed to Allow Synthetic Turf Material to be Adhered Directly to the Aluminum Surface with Appropriate Adhesive and/or Mechanical Fasteners as Determined by Others
d. Assembly Hardware
PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  
SHAFIT – Schutt/Hollywood Anchor Access Frame for Infill Turf shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings. Concrete anchoring foundations to be determined by others based on local soil conditions and building codes. Installer should have a minimum of five (5) baseball/softball equipment installations or similar experience in the previous three (3) years.

    END OF SECTION
Athletic Field Equipment
          11 68 33 
1.9.2020

