ST512 Scorer’s Table 5’W x 12’L
SECTION 11 68 33
Athletic Field Equipment
PART 1
GENERAL 
1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein.  Athletic equipment shall include, but not be limited to:

1. ST512 Scorer’s Table 5’W x 12’L
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section.  Other specification divisions and sections that directly relate to
                       
the work of this section include, but are not limited to:

1. Division 03 – Concrete; Sections: Cast-in-Place Concrete
2. Division 12 – Furnishings; Sections: Other Furnishings 

3. Division 31 – Earthwork; Sections: Excavation and Backfill
                       and Establishment of Sub-Grade Elevations
4. Division 32 – Exterior Improvements; Sections: Athletic and
                       Recreational Surfacing, Concrete and Asphalt

1.03

REFERENCES

A.

Comply with applicable requirements of the following standards. Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.




1.  National Federation of State High School Associations (NFHS)




2.  National Collegiate Athletic Association (NCAA)






3.  World Athletics (WA), Formerly International Association of
                                         Athletics Federations (IAAF)
4.  American Sports Builders Association (ASBA)



5.  Manufacturers Data and Recommended Installation 




     Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data




1.  Provide manufacturers product data prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

B.

Shop Drawings




1.  Provide drawings of the manufacturers recommended 




     installation and foundation requirements prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements. The Manufacturer
                        shall have a current American Sports Builders Association (ASBA)
                        Supplier Certificate of Distinction designation.
1.06

PRODUCT DELIVERY AND STORAGE

A.

Materials delivered to the site shall be examined for damage or defects in 


shipping. Any defects shall be noted and reported to the Owners 



representative. Replacements, if necessary, shall be immediately re-


ordered, so as to minimize any conflict with the construction schedule.  


Sound materials shall be stored above ground under protective cover or 


indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01
ST512 Scorer’s Table 5’W x 12’L
A. 
BASE:  ST512 Scorer’s Table 5’W x 12’L as Manufactured and Supplied
              by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343
www.sportsfieldspecialties.com
B.
COMPONENTS: 
1. ST512 Scorer’s Table 5’W x 12’L:
a. Overall Dimensions: 5’-11 7/8”W x 14’L x 9’-4 15/16”H
b. Fully Welded Aluminum Frame Fabricated of:

i. 2” Square x 1/8” (0.125”) Wall Aluminum Tube
ii. 2” Square x 0.090” Wall Aluminum Tube
iii. 1-1/2” Square x 0.140” Wall Aluminum Tube

iv. 1-1/2” Square x 0.090” Wall Aluminum Tube

v. 3” x 2” x 1/8” (0.125”) Wall Aluminum Rectangular Tube
2. Bench Fabricated of 10”W Aluminum Extrusion Seat and 8”W Aluminum Extrusion Backrest, 10’-8”L
3. Countertop Fabricated of 1/8” (0.125”) Formed Aluminum Sheet, Fully Welded Construction
4. Slip-Resistant Entry/Exit Step with Chrome Plated Steel Grab Handle (Both Sides)
5. Super Durable Powder Coated Finish, Various Standard Colors Available
6. Flooring Fabricated of ¾” (0.75”) Thick Exterior Grade Pressure Treated Plywood Factory Covered with Gray Synthetic Outdoor Carpet
7. 29 Gauge Corrugated Metal Roofing Over Welded Aluminum Frame Roof Structure, Various Standard Colors Available
8. BaseZone® Protective Padding:

a. 19 oz. UV Resistant Premium Outdoor Vinyl, Various Standard Colors Available

b. 3” Thick 1580 Polyurethane Foam

c. Advantech® Composite Sheathing Backer Painted on All Sides
d. Spans Front Face, Front Corners, and Sides

e. Optional Custom Digitally Printed Lettering and/or Graphics Available Upon Request
PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  
All ST512 Scorer’s Tables and Accessories shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings.


    END OF SECTION
Athletic Field Equipment
11 68 33
12.26.2024

