BSS210 10’ Straight Pole Ball Safety Netting System and Accessories
SECTION 11 68 33
Athletic Field Equipment

(Formally Section 11480)

PART 1
GENERAL 
1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein.  Athletic equipment shall include, but not be limited to:

1. BSS210 10’ Straight Pole Ball Safety Netting System and Accessories
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section.  Other specification divisions and sections that directly relate to
                       
the work of this section include, but are not limited to:




1.  Division 03 – Concrete; Sections: Cast-in-Place Concrete 

2.  Division 31 – Earthwork; Sections: Excavation and Backfill and
                                        Establishment of Sub-Grade Elevations




3.  Division 32 – Exterior Improvements; Sections: Athletic and
                                                                Recreational Surfacing, Concrete, Asphalt and
                                                                Site Improvements

1.03

REFERENCES

A.

Comply with applicable requirements of the following standards.  Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.




1.  National Federation of State High School Associations (NFHS)




2.  National Collegiate Athletic Association (NCAA)






3.  International Amateur Athletic Association (IAAF)



4.  American Sports Builders Association (ASBA)



5.  Manufacturers Data and Recommended Installation 




     Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data




1.  Provide manufacturers product data prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

B.

Shop Drawings




1.  Provide drawings of the manufacturers recommended 




     installation and foundation requirements prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements.  The Manufacturer
                        shall have a current American Sports Builders Association (ASBA)
                        Supplier Certificate of Distinction designation.
1.06

PRODUCT DELIVERY AND STORAGE

A.

Materials delivered to the site shall be examined for damage or defects in 


shipping.  Any defects shall be noted and reported to the Owners 



representative.  Replacements, if necessary, shall be immediately re-


ordered, so as to minimize any conflict with the construction schedule.  


Sound materials shall be stored above ground under protective cover or 


indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01

BSS210 10’ Straight Pole Ball Safety Netting System and Accessories
A. 
BASE:   BSS210 10’ Straight Pole Ball Safety Netting System and
              Accessories as manufactured by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343

f. 607-746-8481

                          www.sportsfieldspecialties.com
B.

COMPONENTS:
1. BSS210 Ball Safety Netting System Straight Poles: 
a. 2” Schedule 80 Aluminum Pipe (2.375” O.D.), 12’-8.25”L
b. Standard Powder Coated Black Finish, Various Standard and Custom Powder Coat Finish Color Options Available

2. Ground Sleeves with Welded Base Plates: 

a. 24”L Ground Sleeves


b. Aluminum Tube with Alignment Bolt
3. Net with Perimeter Rope Binding:

a. Overall Dimensions Specified by Customer


b. 1-3/4" Square Mesh


c. #36 Black Nylon


d. Sewn 1/4” Diameter Braided Rope Binding on Perimeter Edges


e. Standard Color is Black
4. Included Accessories:

a. Stainless Steel and/or Galvanized Steel Assembly Hardware


b. Five (5) Fixed Welded 3/16” Aluminum Upper, Lower and
                                        Middle Pole Tabs


c. Secure Snap Clips for Net Attachment


d. 1/8” Diameter Galvanized Wire Rope Black Vinyl Coated to
                3/16” Diameter for Top and Bottom of Net Runs

e. Black Plastic Friction Fit Ground Sleeve Caps


f. Model Specific Hardware Kit and Installation Instructions


5. Optional Accessories:

a. Base Plate Mount Ball Safety Netting Systems


b. Custom Ball Safety Netting System Sizes and Designs Available
                Upon Request


c. Custom Net Mesh Sizes and Colors Available Upon Request


d. Stamped and Sealed Drawings and Calculations by a Licensed
                Professional Engineer of Record in the State of Project Location

PART 3 
EXECUTION

A.01 
INSTALLATION OF EQUIPMENT

A.  
All BSS210 10’ Straight Pole Ball Safety Netting System and Accessories shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings.  Concrete anchoring foundations to be determined by others based on local soil conditions and building codes.  Installer should have a minimum of five (5) ball safety netting system installations or similar experience in the previous three (3) years.
    END OF SECTION
Athletic Field Equipment
11 68 33
01.01.18

