# TNTBUC Tie-Back Tension Ball Safety Netting System with Ultra Cross Netting and Dyneema® Rope

# SECTION 11 68 33	Athletic Field Equipment

(Formally Section 11480)

PART 1	GENERAL

1.01		WORK INCLUDED

A.		Provide all equipment and materials, and do all work necessary to furnish

and install the athletic equipment, as indicated on the drawings and as

specified herein.  Athletic equipment shall include, but not be limited to:

# TNTBUC Tie-Back Tension Ball Safety Netting System with Ultra Cross Netting and Dyneema® Rope

1.02		RELATED WORK

A.		Examine contract documents for requirements that affect work of this

section.  Other specification divisions and sections that directly relate to
                       	the work of this section include, but are not limited to:

1.  Division 03 – Concrete; Sections: Cast-in-Place Concrete

2.  Division 31 – Earthwork; Sections: Excavation and Backfill and
                                        Establishment of Sub-Grade Elevations

3.  Division 32 – Exterior Improvements; Sections: Athletic and
                                                                Recreational Surfacing, Concrete, Asphalt and
                                                                Site Improvements

1.03		REFERENCES

A.		Comply with applicable requirements of the following standards.  Where

these standards conflict with other specified requirements, the most

restrictive requirements shall govern.

1.  National Federation of State High School Associations (NFHS)

2.  National Collegiate Athletic Association (NCAA)

3.  International Association of Athletics Federations (IAAF)

4.  American Sports Builders Association (ASBA)				5.  Manufacturers Data and Recommended Installation

Requirements

1.04		SUBMITTALS

A.		Manufacturers Product Data

1.  Provide manufacturers product data prior to actual field

installation work, for Architects or Owners representatives

review.

B.		Shop Drawings

1.  Provide drawings of the manufacturers recommended

installation and foundation requirements prior to actual field

installation work, for Architects or Owners representatives

review.

1.05		QUALITY ASSURANCE

A.		Manufacturers warranties shall pass to the Owner and certification made

that the product materials meet all applicable grade trademarks or conform 		to industry standards and inspection requirements. The Manufacturer shall
                        have a current American Sports Builders Association (ASBA) Supplier
                        Certificate of Distinction designation.

1.06		PRODUCT DELIVERY AND STORAGE

A.		Materials delivered to the site shall be examined for damage or defects in 			shipping.  Any defects shall be noted and reported to the Owners 				representative.  Replacements, if necessary, shall be immediately re-			ordered, so as to minimize any conflict with the construction schedule.  			Sound materials shall be stored above ground under protective cover or 			indoors so as to provide proper protection.

PART 2	PRODUCTS

2.01	TNTBUC Tie-Back Tension Ball Safety Netting System with Ultra Cross Netting and Dyneema® Rope

A. 	BASE:	  TNTBUC – Tie-Back Tension Ball Safety Netting System with Ultra Cross Netting and Dyneema® Rope as Manufactured and/or Supplied by:

Sportsfield Specialties, Inc.

P.O. Box 231

41155 State Highway 10

Delhi, NY 13753

p. 888-975-3343

f. 607-746-8481

www.sportsfield.com

B.		COMPONENTS:

- Tie-Back Tension Ball Safety Netting System Upright Support Posts and/or Tie-Back Structures – Fabricated, Sized and Configured as Required:

a. Height Above Finish Grade as Required

b. Super Durable Powder Coated Black Finish with Enhanced
                                        Resistance to UV and Fade

c. Ground Sleeve, Base Plate or Permanent Embedment Mount.

d. Stadium Structure can be Utilized if Available (Designed, Fabricated, and Installed by Others)

e. Hot Dipped Galvanized Assembly Hardware - Quantities, Sizes
                and Configurations as Required

2.  Tie-Back Tension Ball Safety Netting System Dyneema® Rope Support Structure:

a. Sizing, Length, Height and Configuration as Required

b. EverSteel™-X Dyneema® Rope:

a. Fiber: Dyneema®

b. Color: Black

c. Construction:

i. Dyneema® Max DM20 Technology

1. Achieves superior creep performance

2. Optimal for long-term static loads

ii. Torque-Free, 12-Strand Single Braid

iii. Extremely Low Stretch and Superior Flex Fatigue and Wear Resistance

d. Elastic Elongation

i. 0.30% Elongation @ 10% of Break Strength

ii. 0.55% Elongation @ 20% of Break Strength

iii. 0.80% Elongation @ 30% of Break Strength

c. EverSteel-X™ Dyneema® Rope - 3/4” Diameter Main
    Horizontal Support, 61,400 lb. Minimum Breaking Strength,
    20,466 lb. Minimum Working Load Limit

d. EverSteel-X™ Dyneema® Rope - 1/2” Diameter

Tie-Back Support, 28,700 lb. Minimum Breaking Strength,
    9,566 lb. Minimum Working Load Limit

e. 7 x 19 GAC Galvanized Aircraft Cable - 1/4” Diameter Vertical
    and Bottom Horizontal Supports, 7,000 lb. Minimum Breaking
    Strength, 2,333 lb. Minimum Working Load Limit

f. Hot Dipped Galvanized Attachment and Assembly Hardware - 
    Quantities, Sizes and Configurations as Required

3.  Tie-Back Tension Ball Safety Netting System Net and Rope Bound  
                 Border:

a. Length, Height and Configuration as Required

b. Ultra Cross Knotless Netting

c. Dyneema® Ultra-High Molecular Weight Polyethylene

(UHMWPE) SK-75 Black Fiber Construction

d. 4 Ply, 1.2 mm (0.0472”) Diameter Twine

e. 95% Open Mesh Area (See-Through Visibility)

f. 58,445 psi Minimum Breaking Strength

g. 30% Maximum Elongation at Break

h. 1-3/4” (44 mm) Square Mesh Size, 0.009 lbs. per Square Foot

i.  4-Strand, Braided, Continuous Monofilament Dyneema® Fiber

j.  Black Multi-Filament Polypropylene Solid Braid Derby Rope
    Sewn Binding on Perimeter Edges - 1/4” Diameter, 530 lb.
    Minimum Breaking Strength

k. Urethane Black Bonded Finish

i. Field Green Finish Available Upon Request

l.  Strong Resistance to Ultraviolet (UV) Light Degradation

m. Excellent Resistance to Chemicals and Water Absorption

4.  Included Accessories:

a. Hot Dipped Galvanized Attachment and Assembly Hardware -             
                                        Quantities, Sizes and Configurations as Required

b. Black Rope for Net Binding Attachment to Rope Support
    Structure – Quantities and Configurations as Required

c. Stamped and Sealed Drawings and Calculations by a Licensed

Professional Engineer of Record in the State of Project Location
d. Model Specific Hardware Kit and Installation Instructions
e. One (1) Year Limited Manufacturer’s Product Warranty

# PART 3 	EXECUTION

3.01		INSTALLATION OF EQUIPMENT

# A.  	All TNTBUC Tie-Back Tension Ball Safety Netting Systems with Ultra Cross Netting and Dyneema® Rope shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings.  Concrete anchoring foundations to be determined by others based on local soil conditions and building codes.  Installer should have a minimum of five (5) ball safety netting system installations or similar experience in the previous three (3) years.

END OF SECTION