FSNSTN – Tension Ball Safety Netting System
SECTION 11 6600
Athletic Equipment

(Formally Section 11480)

PART 1
GENERAL 
1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein.  Athletic equipment shall include, but not be limited to:

1. FSNSTN - Tension Ball Safety Netting System
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section.  Other specification sections that directly relate to the work of 



this section include, but are not limited to:




1.  Section 02200 – Earthwork; Excavation and Backfill and 




     establishment of sub-grade elevations.




2.  Section 02700 – Asphalt, Concrete, and Pavement




3.  Section 02794 – Synthetic Grass & Turf



4.  Section 02793 – Synthetic Field Surfacing




5.  Section 03110 – Cast-in-Place Concrete

1.03

REFERENCES

A.

Comply with applicable requirements of the following standards.  Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.




1.  National Federation of State High School Associations (NFHS)




2.  National Collegiate Athletic Association (NCAA)






3.  International Amateur Athletic Association (IAAF)



4.  American Sports Builders Association (ASBA)



5.  Manufacturers Data and Recommended Installation 




     Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data




1.  Provide manufacturers product data prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

B.

Shop Drawings




1.  Provide drawings of the manufacturers recommended 




     installation and foundation requirements prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements.

1.06

PRODUCT DELIVERY AND STORAGE

A.

Materials delivered to the site shall be examined for damage or defects in 


shipping.  Any defects shall be noted and reported to the Owners 



representative.  Replacements, if necessary, shall be immediately re-


ordered, so as to minimize any conflict with the construction schedule.  


Sound materials shall be stored above ground under protective cover or 


indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01

Tension Ball Safety Netting System
A. 
BASE:
  FSNSTN - Tension Ball Safety Netting System as Manufactured and/or Supplied by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343

f. 607-746-8481

www.sportsfieldspecialties.com
B.

COMPONENTS:

1. Tension Ball Safety Netting System Upright Support Posts and 

      Tie-Back Structures – Fabricated, Sized and Configured as Required

A. Height Above Finish Grade as Required


B. Powder Coated Finish (Various Colors Available)


C. Ground Sleeve, Base Plate or Permanent Embedment Mount


D. Stainless Steel and/or Hot Dipped Galvanized Assembly
      
     Hardware - Quantities, Sizes and Configurations as Required

2. Tension Ball Safety Netting System galvanized Wire Rope Support

    Structure:


A. Length, Height and Configuration as Required

B. Galvanized Steel 6 X 25 Wire Rope – 3/4” Diameter Main 

    Horizontal Support, 53,000 lb. Minimum Breaking Strength, 

    17,667 lb. Minimum Working Load Limit

C. Galvanized Steel 7 X 19 Wire Rope – 1/4” Diameter Main 

    Horizontal Support, 7,000 lb. Minimum Breaking Strength, 

    2,333 lb. Minimum Working Load Limit

D. Stainless Steel and/or Hot Dipped Galvanized Assembly
                Hardware - Quantities, Sizes and Configurations as Required

3. Tension Ball Safety Netting System Net and Rope Bound Border:


A. Length, Height and Configuration as Required


B. #36 Twisted Knotted Netting




C. 100% Nylon Twisted, Knotted, 1-3/4" square mesh; UV treated

                                        and weather resistant Construction.


D. 2.6 mm (0.102”) Diameter Twine

E. 345 lb. Breaking Strength of Netting

F. 1-3/4” (44 mm) Square Mesh Size, 0.0425 lbs. per Square Foot

4. Accessories:


A. Stainless Steel and/or Hot Dipped Galvanized Assembly
                                         Hardware - Quantities, Sizes and Configurations as Required

B. Stamped and Sealed Drawings and Calculations by a Licensed

     Professional Engineer of Record in the State of Project Location

PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  

All athletic equipment shall be installed as recommended with 



manufacturer's written directions, and as indicated on the drawings.



    END OF SECTION
Athletic Equipment
114 6600
07.13.12

