FSNS64040 - Football Safety Netting System, 40’H x 40’W  
SECTION 11 6600
Athletic Equipment - Field

(Formally section 11480)

PART 1
GENERAL 
1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein.  Athletic equipment shall include, but not be limited to:




1.  Football Safety Netting System
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section.  Other specification sections that directly relate to the work of 



this section include, but are not limited to:




1.  Section 02200 – Earthwork; Excavation and Backfill and 




     establishment of sub-grade elevations.




2.  Section 02700 – Asphalt, Concrete, and Pavement




3.  Section 02794 – Synthetic Grass & Turf




4.  Section 02793 – Synthetic Field Surfacing




5.  Section 03110 – Cast-in-Place Concrete

1.03

REFERENCES

A.

Comply with applicable requirements of the following standards.  Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.




1.  National Federation of State High School Associations (NFHS)




2.  National Collegiate Athletic Association (NCAA)






3.  International Amateur Athletic Association (IAAF)



4.  American Sports Builders Association (ASBA)




5.  Manufacturers Data and Recommended Installation 




     Requirements
1.04

SUBMITTALS

A.

Manufacturers Product Data

1. Provide manufacturers product data prior to actual field installation work, for Architects or Owners representatives review.

B.

Shop Drawings

1. Provide drawings of the manufacturers recommended installation and foundation requirements prior to actual field installation work, for Architects or Owners representatives review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements.

1.06

PRODUCT DELIVERY AND STORAGE

A.

Materials delivered to the site shall be examined for damage or defects in 


shipping.  Any defects shall be noted and reported to the Owners 



representative.  Replacements, if necessary, shall be immediately re-


ordered, so as to minimize any conflict with the construction schedule.  


Sound materials shall be stored above ground under protective cover or 


indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01

Football Safety Netting System, 40’H x 40’W
A. 

BASE:  FSNS64040 Football Safety Netting System as manufactured by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343

www.sportsfieldspecialties.com
B.

COMPONENTS:
1. Uprights:
a. 6” Schedule 40 Steel Poles (6.625” O.D.)

b. 45’ Overall Length; 41’ Above Grade
c. Black Super Durable Powder Coated Finish

d. Structural Welded Tabs for Hardware Connections and Installation Aid

2. Ground Sleeves:
a. 7” x .109” Wall Steel Pipe

b. 4’ Length

c. Alignment Bolt
d. Welded Base Plate

3. Netting:

a. #36 Twisted Knotted Nylon Netting
i. Size: 40’H x 40’W
ii. 4” (102 mm) Square Mesh Size
iii. 100% Nylon Construction

iv. 2.6 mm (0.1023”) Diameter Twine
v. 96% Open Mesh Area (See-Through Visibility)

vi. 13,363 psi Minimum Breaking Strength 

vii. Black Multi-Filament Polypropylene Solid Braid Derby Rope Sewn Binding on Perimeter Edges - 1/4” Diameter, 530 lb. Minimum Breaking Strength

viii. UV and Weather Treated



4. Accessories:

a. Retractable Pulley System for Ease of Raising and Lowering Net
b. Vertical Slide Cable and Associated Hardware

PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  

All athletic equipment shall be installed as recommended with 



manufacturer's written directions, and as indicated on the drawings.



    END OF SECTION
Athletic Equipment
11 6600
6.6.2019

