SG824INT 8’ x 24’ International Round Post Soccer Goals and Accessories
SECTION 11 68 33.23
Athletic Field Equipment – Soccer and Field Hockey
(Formally Section 11480)                     Equipment
PART 1
GENERAL 
1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein.  Athletic equipment shall include, but not be limited to:

1.  SG824INT 8’ x 24’ International Round Post Soccer Goals and
     Accessories - 8’H x 24’L x 6’B x 6’D
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section.  Other specification divisions and sections that directly relate to
                       
the work of this section include, but are not limited to:




1.  Division 03 – Concrete; Sections: Cast-in-Place Concrete 

2.  Division 31 – Earthwork; Sections: Excavation and Backfill and
                                        Establishment of Sub-Grade Elevations




3.  Division 32 – Exterior Improvements; Sections: Athletic and
                                                                Recreational Surfacing, Concrete and Asphalt

1.03

REFERENCES

A.

Comply with applicable requirements of the following standards.  Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.




1.  National Federation of State High School Associations (NFHS)




2.  National Collegiate Athletic Association (NCAA)






3.  International Association of Athletics Federations (IAAF)



4.  Fédération Internationale de Football Association (FIFA)



5.  ASTM International

6.  American Sports Builders Association (ASBA)


7.  Manufacturers Data and Recommended Installation 




     Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data




1.  Provide manufacturers product data prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

B.

Shop Drawings




1.  Provide drawings of the manufacturers recommended 




     installation and foundation requirements prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements.  The Manufacturer
                        shall have a current American Sports Builders Association (ASBA)
                        Supplier Certificate of Distinction designation.
1.06

PRODUCT DELIVERY AND STORAGE

A.

Materials delivered to the site shall be examined for damage or defects in 


shipping.  Any defects shall be noted and reported to the Owners 



representative.  Replacements, if necessary, shall be immediately re-


ordered, so as to minimize any conflict with the construction schedule.  


Sound materials shall be stored above ground under protective cover or 


indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01

SG824INT 8’ x 24’ International Round Post Soccer Goals and
                       
Accessories


A. 

BASE:  SG824INT 8’ x 24’ International Round Post Soccer Goals and
                                     Accessories as Manufactured and/or Supplied by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343

f. 607-746-8481

www.sportsfieldspecialties.com
B.

COMPONENTS:
1. Crossbar and Uprights Fabricated of 6061 T6 Aluminum Pipe, 3.5” Schedule 40 Round (4” O.D.), Having the Following Attributes:

a. Inside Width: 24’

b. Inside Height: 8’

c. Super Durable Powder Coated White Finish with Enhanced Resistance to UV and Fade 

d. Welded Aluminum Net Clips, Guaranteed for Life

e. 24” Aluminum Ground Sleeves and Ground Sleeve Caps

2. Ground Bar Fabricated of 6061 T6 Extruded Aluminum Tube, 2” Square x 0.125” Wall, Having the Following Attributes:

a. Super Durable Powder Coated White Finish with Enhanced Resistance to UV and Fade 

b. Pivot Attachment with Pull Pin
i. Retractable Ground Bar Permits Raised Mowing Position
3. Rear Net Support Uprights Fabricated of 2” Schedule 80 Aluminum Pipe (2.375” O.D.), Having the Following Attributes:

a. 9’-3” Height Above Finish Grade

b. Super Durable Powder Coated Black Finish with

Enhanced Resistance to UV and Fade 

c. 24” Aluminum Ground Sleeves and Ground Sleeve Caps

d. Includes Pulleys, Tie-Back Tethers, and Rope Cleats
4. Additional:

a. Welded Aluminum Net Clips, Guaranteed for Life

b. 4mm Braided, Knotless White High Tenacity
Polypropylene Soccer Net with Rope Bound
Perimeter and 5” Hexagonal Mesh – 8’H x 24’L x 6’B x 6’D

c. Model Specific Hardware Kit and Installation
Instructions

d. Five (5) Year Limited Manufacturer’s Product
Warranty
PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  

All SG824INT 8’ x 24’ International Round Post Soccer Goals and
            Accessories shall be installed as recommended per manufacturer's written
      
instructions and as indicated on the drawings.  
    END OF SECTION
Athletic Field Equipment -
                  11 68 33.23
          
         
          01.01.18
Soccer and Field Hockey Equipment

      

