# GP830COLGHPL Gooseneck Hydraulically Hinged College Plate Mount Football Goal Posts and Accessories

# SECTION 11 68 33.13 	Athletic Field Equipment - Football Field Equipment

# (Formally Section 11480)

PART 1	GENERAL

1.01		WORK INCLUDED

A.		Provide all equipment and materials, and do all work necessary to furnish

and install the athletic equipment, as indicated on the drawings and as

specified herein.  Athletic equipment shall include, but not be limited to:

- GP830COLGHPL Gooseneck Hydraulically Hinged College Plate Mount Football Goal Posts and Accessories

1.02		RELATED WORK

A.		Examine contract documents for requirements that affect work of this

section.  Other specification divisions and sections that directly relate to
                       	the work of this section include, but are not limited to:

1.  Division 03 – Concrete; Sections: Cast-in-Place Concrete

2.  Division 31 – Earthwork; Sections: Excavation and Backfill and
                                        Establishment of Sub-Grade Elevations

3.  Division 32 – Exterior Improvements; Sections: Athletic and
                                                                Recreational Surfacing, Concrete and Asphalt

1.03		REFERENCES

A.		Comply with applicable requirements of the following standards.  Where

these standards conflict with other specified requirements, the most

restrictive requirements shall govern.

1.  National Federation of State High School Associations (NFHS)

2.  World Athletics (Formerly IAAF)

3.  National Collegiate Athletic Association (NCAA)

4.  National Football League (NFL)

5.  American Sports Builders Association (ASBA)

6.  Manufacturer’s Data and Recommended Installation

Requirements

1.04		SUBMITTALS

A.		Manufacturers Product Data

1.  Provide manufacturers product data prior to actual field

installation work, for Architects or Owners representatives

review.

B.		Shop Drawings

1.  Provide drawings of the manufacturers recommended

installation and foundation requirements prior to actual field

installation work, for Architects or Owners representatives

review.

1.05		QUALITY ASSURANCE

A.		Manufacturers warranties shall pass to the Owner and certification made

that the product materials meet all applicable grade trademarks or conform 		to industry standards and inspection requirements.  The Manufacturer
                        shall have a current American Sports Builders Association (ASBA)
                        Supplier Certificate of Distinction designation.

1.06		PRODUCT DELIVERY AND STORAGE

A.		Materials delivered to the site shall be examined for damage or defects in 			shipping.  Any defects shall be noted and reported to the Owners 				representative.  Replacements, if necessary, shall be immediately re-			ordered, so as to minimize any conflict with the construction schedule.  			Sound materials shall be stored above ground under protective cover or 			indoors so as to provide proper protection.

PART 2	PRODUCTS

2.01		GP830COLGHPL Gooseneck Hydraulically Hinged College Plate
                        Mount Football Goal Posts and Accessories

A. 		BASE:  GP830COLGHPL Gooseneck Hydraulically Hinged College Plate
                                      Mount Football Goal Posts and Accessories as Manufactured
                                      and/or Supplied by:

Sportsfield Specialties, Inc.

P.O. Box 231

41155 State Highway 10

Delhi, NY 13753

p. 888-975-3343

f. 607-746-8481

www.sportsfield.com

B.		COMPONENTS:

- Two Piece Hydraulically Hinged Gooseneck Support Consisting of the Following Components:

  - Upper and Lower Gooseneck Fabricated of 6” Schedule 40 Aluminum Pipe (6.625” O.D.) with the Upper Gooseneck Rolled to a 3’ Radius and 8’ Offset

  - ¾” Aluminum Mounting Plates with ¾” Aluminum Gussets

  - ¾” A572 Steel Hinge Plates with a Black Powder Coated Finish

  - ½” Dia. Grade 8 Zinc Yellow-Chromate Plated Steel Pivot Pin

  - Hydraulic Cylinder, 2.50” Bore x 1.25” Rod, 12” Stroke, 3000 PSI

  - ½” Dia. x 3.50” Long 17-4 Stainless Steel Locking Pull Pin, Includes Padlock

- Removable Hydraulic Compartment: Fabricated of 0.125” Aluminum Sheet with Removable 0.125” Aluminum Cover with Handle. Consists of the Following Components:

  - 4.25 Cup Capacity Hydraulic Fluid Tank Fabricated of 0.125” Aluminum Sheet with Plastic Vented Cap

  - 0.097 in3, 4000 PSI Hydraulic Gear Pump for Raising Goal

  - 3-Way Directional Control Valve with Adjustable Relief Valve for Lowering Goal, 3000 PSI Max

  - Adjustable Flow Control Valve, 5000 PSI Max

  - 3/8” I.D. Hydraulic Hose with Quick Disconnect Coupling, 4000 PSI Max

  - Assorted Steel and Stainless Steel Fittings, Couplers, and Mounting Hardware

  - 4.00 Cups of AW-46 Hydraulic Fluid

3.  Crossbar: Fabricated of 6” Schedule 40 Aluminum Pipe (6.625”
     O.D.)

a.   Length: 18’-6” – College

b.   Includes Patented AdjustRight® feature allowing for
      easy installation through the adjustment of an internal
      locking rotating sleeve at both the gooseneck/crossbar
      and upright/crossbar connections.  This adjustment can
      easily be repeated throughout the life of the football
      goal post ensuring proper alignment of all components
      for years of competition and all with the added benefit
      of no exposed hardware on the face of the goal.
      Thermal arc sprayed internal textured mating surfaces
      and anti-vibration enhancements such as serrated
      washers and nyloc coated bolt ends ensure the
      AdjustRight® Football Goal Posts remain in position.

4.  Uprights: Fabricated of Extruded 6061-T6 Aluminum Tube (4”
     O.D.) with Rigid Wire Loop Welded to Upper End

a.   Length: 30’

5.  Super Durable Powder Coated Finish

- Color: Yellow or White

6.  Installation Package Consisting of the Following Components:

- Base Plate Mounting Kit

- Optional Access Frame Kit: 1/8” (0.125”) Aluminum Construction with 1” PVC Drain Stub, Includes Two (2) Half Moon Filler Plugs, Optional Full Size Filler Plug and SG2S® Patented Soccer Goal Rear Bottom Ground Bar Retractable Safety Clamp System Available, Use GFAFIT for Synthetic Turf Installation Applications and GPAFNG for Natural Grass Installation Applications

7.  Included Accessories:

- Directional Wind Flags

- Touch-up Paint

- Model Specific Hardware Kit and Installation Instructions

8.  Optional Accessories:

a.   Football Goal Post Pads: 18 oz. Vinyl with Polyester
      Scrim and Vertically Sewn in Hook and Loop
      Securement, Standard 6’ in Height, Various Standard
      Vinyl Colors Available, Custom Digitally Printed
      Lettering and/or Graphics Available Upon Request

1.  GPPRGH – Round, Fully Encased Vinyl,
                 18” O.D. and 7” I.D. Polyurethane Foam Core

2.  GPPRGHDG – Custom Digitally Printed
                 Graphics

b.   Football End Zone Pylons: Set of Four (4) Orange
                                          Vinyl Covered Foam Football End Zone Pylons with
                                          Self-Standing Weighted Bases, 18”H x 4”L x 4”W

# PART 3 	EXECUTION

3.01		INSTALLATION OF EQUIPMENT

A.  	All GP830COLGHPL Gooseneck Hydraulically Hinged College Plate Mount Football Goal Posts and Accessories shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings.  Concrete anchoring foundations to be determined by others based on local soil conditions and building codes.  Installer should have a minimum of five (5) football goal post installations or similar experience in the previous three (3) years.

END OF SECTION