GP630ADJ Ground Sleeve Insert Adjustable College/High School Football Goal Posts and Accessories
SECTION 11 68 33.13 
Athletic Field Equipment - Football Field Equipment

 (Formally Section 11480)

PART 1
GENERAL 
1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein.  Athletic equipment shall include, but not be limited to:




1.  GP630ADJ Ground Sleeve Insert Adjustable College/High
                                         School Football Goal Posts and Accessories
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section.  Other specification divisions and sections that directly relate to
                       
the work of this section include, but are not limited to:




1.  Division 03 – Concrete; Sections: Cast-in-Place Concrete 

2.  Division 31 – Earthwork; Sections: Excavation and Backfill and
                                        Establishment of Sub-Grade Elevations




3.  Division 32 – Exterior Improvements; Sections: Athletic and
                                                                Recreational Surfacing, Concrete and Asphalt

1.03

REFERENCES

A.

Comply with applicable requirements of the following standards.  Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.




1.  National Federation of State High School Associations (NFHS)




2.  International Association of Athletics Federations (IAAF)




3.  National Collegiate Athletic Association (NCAA)




4.  National Football League (NFL)

5.  American Sports Builders Association (ASBA)




6.  Manufacturer’s Data and Recommended Installation 




     Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data




1.  Provide manufacturers product data prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

B.

Shop Drawings




1.  Provide drawings of the manufacturers recommended 




     installation and foundation requirements prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements.  The Manufacturer
                        shall have a current American Sports Builders Association (ASBA)
                        Supplier Certificate of Distinction designation.
1.06

PRODUCT DELIVERY AND STORAGE

A.

Materials delivered to the site shall be examined for damage or defects in 


shipping.  Any defects shall be noted and reported to the Owners 



representative.  Replacements, if necessary, shall be immediately re-


ordered, so as to minimize any conflict with the construction schedule.  


Sound materials shall be stored above ground under protective cover or 


indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01

GP630ADJ Ground Sleeve Insert Adjustable College/High School
                        Football Goal Posts and Accessories
A. 

BASE:  GP630ADJ Ground Sleeve Insert Adjustable College/High School
                                      Football Goal Posts and Accessories as Manufactured and/or
                                      Supplied by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343
f. 607-746-8481

www.sportsfieldspecialties.com
B.

COMPONENTS:
1. Single Ground Sleeve Insert Gooseneck Support: Fabricated of 6” Schedule 40 Aluminum Pipe (6.625” O.D.), 5’ Radius, 6’ Offset, Custom Offsets Available
2. Ground Sleeve: Fabricated of 8” Schedule 40 Steel Pipe (8” I.D.)
a.   Length: 60”
3.  Crossbar: Fabricated of 6” Schedule 40 Aluminum Pipe (6.625”
     O.D.)


a.   Length: Adjustable from 18’-6” to 23’-4”
                  (College/High School)
b.   Includes Patented AdjustRight® feature allowing for
      easy installation through the adjustment of an internal
      locking rotating sleeve at both the gooseneck/crossbar
      and upright/crossbar connections.  This adjustment can
      easily be repeated throughout the life of the football
      goal post ensuring proper alignment of all components
      for years of competition and all with the added benefit
      of no exposed hardware on the face of the goal.
      Thermal arc sprayed internal textured mating surfaces
      and anti-vibration enhancements such as serrated
      washers and nyloc coated bolt ends ensure the
      AdjustRight® Football Goal Posts remain in position.
4.  Uprights: Fabricated of Extruded 6061-T6 Aluminum Tube (4”
     O.D.) with Rigid Wire Loop Welded to Upper End
a.   Length: 30’


5.  Powder Coated Finish: Yellow or White
6.  Installation Package Consisting of the Following Components:

a. Ground Sleeve
b. Optional Access Frame Kit: 1/8” (0.125”) Aluminum Construction with 1” PVC Drain Stub, Includes Two (2) Half Moon Filler Plugs, Optional Full Size Filler Plug and SG2S® Patented Soccer Goal Rear Bottom Ground Bar Retractable Safety Clamp System Available, Use GFAFIT for Synthetic Turf Installation Applications and GPAFNG for Natural Grass Installation Applications
7.  Included Accessories:

a. Directional Wind Flags
b. Touch-up Paint (Powder Coat Finish Specific)
c. Model Specific Hardware Kit and Installation Instructions
8.  Optional Accessories:

a.   Football Goal Post Pads: 18 oz. Vinyl with Polyester
      Scrim and Vertically Sewn in Hook and Loop
      Securement, Standard 6’ in Height, Various Standard
      Vinyl Colors Available, Custom Digitally Printed
      Lettering and/or Graphics Available Upon Request 

1.  GPPR – Round, Fully Encased Vinyl,
                 18” O.D. and 7” I.D. Polyurethane Foam Core

2.  GPPRDG – Custom Digitally Printed Graphics




b.   Football End Zone Pylons: Set of Four (4) Orange
                                          Vinyl Covered Foam Football End Zone Pylons with
                                          Self-Standing Weighted Bases, 18”H x 4”L x 4”W    
PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  
All GP630ADJ Ground Sleeve Insert Adjustable College/High School Football Goal Posts and Accessories shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings.  Concrete anchoring foundations to be determined by others based on local soil conditions and building codes.  Installer should have a minimum of five (5) football goal post installations or similar experience in the previous three (3) years.

    END OF SECTION
Athletic Field Equipment -
             11 68 33.13            
01.01.18
Football Field Equipment


