GPKS30COLRGH College Rotating and Gooseneck Hinged GoalPak® Combination Football/Soccer Goal System and Accessories
SECTION 11 68 33.13 
Athletic Field Equipment - Football Field Equipment

SECTION 11 68 33.23
Athletic Field Equipment – Soccer and Field Hockey
(Formally Section 11480)                     Equipment
PART 1
GENERAL 
1.01

WORK INCLUDED

A.

Provide all equipment and materials, and do all work necessary to furnish 



and install the athletic equipment, as indicated on the drawings and as 



specified herein.  Athletic equipment shall include, but not be limited to:

1.  GPKS30COLRGH College Rotating and Gooseneck Hinged
     GoalPak® Combination Football/Soccer Goal System and
     Accessories
1.02

RELATED WORK

A.

Examine contract documents for requirements that affect work of this 



section.  Other specification divisions and sections that directly relate to
                       
the work of this section include, but are not limited to:




1.  Division 03 – Concrete; Sections: Cast-in-Place Concrete 

2.  Division 31 – Earthwork; Sections: Excavation and Backfill and
                                        Establishment of Sub-Grade Elevations




3.  Division 32 – Exterior Improvements; Sections: Athletic and
                                                                Recreational Surfacing, Concrete and Asphalt

1.03

REFERENCES

A.

Comply with applicable requirements of the following standards. Where 



these standards conflict with other specified requirements, the most 



restrictive requirements shall govern.




1.  National Federation of State High School Associations (NFHS)




2.  National Collegiate Athletic Association (NCAA)

3.  World Athletics (WA), Formerly International Association of
     Athletics Federations (IAAF)



4.  Fédération Internationale de Football Association (FIFA)
5.  ASTM International
6.  American Sports Builders Association (ASBA)




7.  Manufacturer’s Data and Recommended Installation 




     Requirements

1.04

SUBMITTALS

A.

Manufacturers Product Data




1.  Provide manufacturers product data prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

B.

Shop Drawings




1.  Provide drawings of the manufacturers recommended 




     installation and foundation requirements prior to actual field 




     installation work, for Architects or Owners representatives 




     review.

1.05

QUALITY ASSURANCE

A.

Manufacturers warranties shall pass to the Owner and certification made 



that the product materials meet all applicable grade trademarks or conform 

to industry standards and inspection requirements. The Manufacturer
                        shall have a current American Sports Builders Association (ASBA)
                        Supplier Certificate of Distinction designation.
1.06

PRODUCT DELIVERY AND STORAGE

A.

Materials delivered to the site shall be examined for damage or defects in 


shipping.  Any defects shall be noted and reported to the Owners 



representative.  Replacements, if necessary, shall be immediately re-


ordered, so as to minimize any conflict with the construction schedule.  


Sound materials shall be stored above ground under protective cover or 


indoors so as to provide proper protection.
PART 2
PRODUCTS 
2.01
GPKS30COLRGH College Rotating and Gooseneck Hinged GoalPak® Combination Football/Soccer Goal System and Accessories
A. 

BASE:  GPKS30COLRGH College Rotating and Gooseneck Hinged
                                      GoalPak® Combination Football/Soccer Goal System and
                                      Accessories as Manufactured and/or Supplied by:
Sportsfield Specialties, Inc.  

P.O. Box 231 

41155 State Highway 10





Delhi, NY 13753   

p. 888-975-3343
www.sportsfieldspecialties.com
B.

COMPONENTS:
A. GP830COLRGH Rotating High School Football Goal Posts:

1. Rotating Base Plate Mounting Kit
2. Two Piece Hydraulically Hinged Gooseneck Support Consisting of the Following Components:
a. Upper and Lower Gooseneck Fabricated of 6” Schedule 40 Aluminum Pipe (6.625” O.D.) with the Upper Gooseneck Rolled to a 3’ Radius and 8’ Offset
b. ¾” Aluminum Mounting Plates with ¾” Aluminum Gussets
c. ¾” A572 Steel Hinge Plates with a Black Powder Coated Finish
d. ½” Dia. Grade 8 Zinc Yellow-Chromate Plated Steel Pivot Pin
e. Hydraulic Cylinder, 2.50” Bore x 1.25” Rod, 12” Stroke, 3000 PSI
f. ½” Dia. x 3.50” Long 17-4 Stainless Steel Locking Pull Pin, Includes Padlock
3. Removable Hydraulic Compartment: Fabricated of 0.125” Aluminum Sheet with Removable 0.125” Aluminum Cover with Handle. Consists of the Following Components:
a. 4.25 Cup Capacity Hydraulic Fluid Tank Fabricated of 0.125” Aluminum Sheet with Plastic Vented Cap
b. 0.097 in3, 4000 PSI Hydraulic Gear Pump for Raising Goal
c. 3-Way Directional Control Valve with Adjustable Relief Valve for Lowering Goal, 3000 PSI Max
d. Adjustable Flow Control Valve, 5000 PSI Max
e. 3/8” I.D. Hydraulic Hose with Quick Disconnect Coupling, 4000 PSI Max
f. Assorted Steel and Stainless Steel Fittings, Couplers, and Mounting Hardware
g. 4.00 Cups of AW-46 Hydraulic Fluid
4.  Crossbar: Fabricated of 6” Schedule 40 Aluminum Pipe (6.625”
     O.D.)


a.   Length: 18’-6” – College
b.   Includes Patented AdjustRight® feature allowing for
      easy installation through the adjustment of an internal
      locking rotating sleeve at both the gooseneck/crossbar
      and upright/crossbar connections.  This adjustment can
      easily be repeated throughout the life of the football
      goal post ensuring proper alignment of all components
      for years of competition and all with the added benefit
      of no exposed hardware on the face of the goal.
      Thermal arc sprayed internal textured mating surfaces
      and anti-vibration enhancements such as serrated
      washers and nyloc coated bolt ends ensure the
      AdjustRight® Football Goal Posts remain in position.
5.  Uprights: Fabricated of Extruded 6061-T6 Aluminum Tube (4”
     O.D.) with Rigid Wire Loop Welded to Upper End
a.   Length: 30’
6.  Super Durable Powder Coated Finish with Enhanced Resistance

     to UV and Fade
a. Color: Yellow or White
7.  Installation Package Consisting of the Following Components:

a. Rotating Base Plate Mounting Kit
b. Access Frame Kit: 1/8” (0.125”) Aluminum Construction with 1” PVC Drain Stub, Includes Two (2) Half Moon Filler Plugs and SG2S® Patented Soccer Goal Rear Bottom Ground Bar Retractable Safety Clamp System, Use SG2SGPR30 for Synthetic Turf Installation Applications and SG2SGPRNG30 for Natural Grass Installation Applications, Optional Full Size Filler Plug Available
8.  Included Accessories:

a. Directional Wind Flags
b. Touch-up Paint (Powder Coat Finish Specific)
c. Model Specific Hardware Kit and Installation Instructions
9.  Optional Accessories:

a.   Football Goal Post Pads: 18 oz. Vinyl with Polyester
      Scrim and Vertically Sewn in Hook and Loop
      Securement, Standard 6’ in Height, Various Standard
      Vinyl Colors Available, Custom Digitally Printed
      Lettering and/or Graphics Available Upon Request 

1.  GPPR – Round, Fully Encased Vinyl,
                 18” O.D. and 7” I.D. Polyurethane Foam Core

2.  GPPRDG – Custom Digitally Printed Graphics




b.   Football End Zone Pylons: Set of Four (4) Orange
                                          Vinyl Covered Foam Football End Zone Pylons with
                                          Self-Standing Weighted Bases, 18”H x 4”L x 4”W   
B. SG824S 8’ x 24’ Regulation Size Square Post Soccer Goals:




1.  Top Crossbar Fabricated of 6061-T6 Extruded Aluminum 

 Tube Having the Following Attributes:

a. Length: 24’ - Regulation Size

b. 4” Square Crossbar, 3/16” (.1875”) Wall Thickness


    c.   Super Durable Powder Coated White Finish with
                                        Enhanced Resistance to UV and Fade


    d.   3/16” (0.1875”) Thick Formed Aluminum Channel
                                        Crossbar Attachment Brackets with Welded Tap
                                        Blocks, Mill Finish

2.  One Piece End Frame Construction Fabricated of 6061-T6
                 Extruded Aluminum Tube Having the Following Attributes:


     a. 
4” Square Corner Post, 8’H, 3/16” (.1875”) Wall
                    Thickness

b. 
Rolled Side Frame, 2” x 3” x 0.125” Thick Wall,
              
TIG Welded to Corner Upright Posts


     c. 
Radius Backside Corners

     d. 
Super Durable Powder Coated White Finish with
                     
Enhanced Resistance to UV and Fade 

3.  Rear Bottom Ground Bar Fabricated of 6061-T6 Extruded
                 Aluminum Tube Having the Following Attributes:

a. 2” x 2” x 0.25” Thick Wall with Welded ½” Aluminum End Plates

b. Super Durable Powder Coated White Finish with
Enhanced Resistance to UV and Fade

4.  Included Accessories:


     a. 
Welded Aluminum Net Clips with Lifetime
                                    
Guarantee 

b. 
5mm Braided, Knotless White High Tenacity
Polypropylene Soccer Net with Rope Bound
Perimeter and 4” Square Mesh – 8.2’H x 24.4’L x 4.3’B x 8.6’D

c. 
Model Specific Hardware Kit and Installation
    
Instructions

d. 
Ground Stake Storage Compartments

e. 
All SG824S 8’ x 24’ Regulation Size Square Post
Soccer Goals Meet and Exceed Current ASTM F2950-14 Standard Safety and Performance Specification for Soccer Goals and F1938-98 Standard Guide for Safer Use of Movable Soccer Goals

 f.   
Five (5) Year Limited Manufacturer’s Product
          
Warranty

C. Included Components:

1.    SGMobile® SGMKS - Internal Soccer Goal Portable
                   Mobility Wheel Kit and Handle:




    a. 
Welded 13 Gauge Stainless Steel Frame

      b. 
Ultra High Molecular Weight Polyethylene
                                
(UHMWPE) Plastic Wheel

      c. 
Stainless Steel Hardware

     d. 
Roll Formed Stainless Steel Rod with Stainless
  
Steel Spring and Cushioned Rubber Handle  

2.    SG2S® - Patented Soccer Goal Safety System:

     
     a. 
Rear Bottom Ground Bar Retractable Safety Clamp
                                             
Fabricated of 3/16” (0.187”) Aluminum




     b. 
Super Durable Powder Coated White Finish with
                                                         
Enhanced Resistance to UV and Fade



     c. 
Stainless Steel Assembly Hardware




     d. 
Access Frame and Cover Fabricated of 1/8”
 



(0.125”) Aluminum with Gasket Seal and 1” PVC
              

Drain Stub

      e. 
13 Gauge Stainless Steel Pivot Bar 


                   f.  
Galvanized Steel Anchoring Hardware


g.
Use SG2SGPR30 for Synthetic Turf Installation
                                                
Applications and SG2SGPRNG30 for Natural Grass
                             

Installation Applications

PART 3 
EXECUTION

3.01

INSTALLATION OF EQUIPMENT

A.  
All GPKS30COLRGH College Rotating and Gooseneck Hinged GoalPak® Combination Football/Soccer Goal System and Accessories shall be installed as recommended per manufacturer's written instructions and as indicated on the drawings.  Concrete anchoring foundations to be determined by others based on local soil conditions and building codes.  Installer should have a minimum of five (5) football goal post installations or similar experience in the previous three (3) years.


    END OF SECTION
Athletic Field Equipment -
             11 68 33.13            
08.22.23
Football Field Equipment and                     11 68 33.23
Soccer and Field Hockey Equipment


